Project Report on

Embedded Deep Learning on Microcontrollers

A Practical Implementation of FFNN on Arduino UNO

Submitted by

Pratham Panchal-20220702066

Course Name: Deep Learning with Tensorflow & Keras (Industry elective-3)

B.Tech CSE - AIML

VI Semester – Dec 2024 – Apr 2025

Dec/Apr 2025

1. Introduction

Embedded Deep Learning (DL) refers to the practice of implementing neural networks directly on low-resource hardware, such as microcontrollers. This enables small, power-efficient devices to perform intelligent tasks without needing cloud connectivity or powerful CPUs.

Embedded DL is particularly valuable for solving problems that involve non-linear behaviors—issues that are traditionally difficult or inefficient to solve using conventional algorithms. It brings intelligence directly into the hardware, enhancing functionality in edge devices.

This project explores the implementation of a Feedforward Neural Network (FFNN) on the ATmega328p microcontroller (commonly known as Arduino UNO), enabling a complete offline inference system trained in Python and executed in C++ on the microcontroller.

2. Objective of This Project

The main objective of this project is to create a minimal framework that:

- Allows for the creation and training of an FFNN in Python.

- Stores the trained parameters in a structured text file and a .h file compatible with Arduino.

- Enables direct execution of the neural network on the Arduino UNO without the use of any external deep learning libraries.

- Demonstrates embedded inference capability using a sample regression model (house price prediction).

This framework also accounts for the memory and processing limitations of the target hardware and aims to deliver efficient runtime performance.

3. Applications of This Project

This FFNN-based embedded system can be used in several real-world applications where conventional algorithms fall short due to non-linear behavior. Some feasible and practical examples include:

1. Smart DC Motor Drivers

- Trained to predict safe control signals by considering voltage drop, current draw, and RPM.

- Helps maintain a more linear relationship between control input (e.g., PWM) and actual output (speed or torque).

- Eliminates the need for manual PID tuning.

2. Battery Health Estimation

- Predicts voltage drop or battery degradation based on usage data, temperature, and load.

- Useful for power management in drones, remote sensors, and portable electronics.

3. Sensor Error Compensation

- Corrects non-linear errors in cheap sensors such as thermistors or IR distance sensors.

- Provides calibrated sensor outputs without complex equations or lookup tables.

4. Environmental Monitoring

- Predicts environmental states like "safe" or "hazardous" from multiple sensor readings (e.g., gas concentration, temperature, humidity).

- Replaces static threshold-based alert systems with a more adaptive and intelligent model.

4. System Design & Implementation

The project consists of two main components:

A. Python-Based Training System

- Defines and trains a fully connected FFNN using supervised learning (with Mean Squared Error loss and ReLU activation).

- Stores the model parameters (weights and biases) in both .txt format (for readability) and .h format for Arduino.

- The .h file is automatically formatted to use PROGMEM and define variables for feedforward computation.

B. Arduino Inference System

- Written in standard C++ using Arduino IDE.

- Loads parameters from the .h file.

- Implements a custom feed_forward() function to compute the output from input neurons using the stored weights and biases.

- Does not rely on vectors or external libraries—suitable for low RAM usage.

The system was demonstrated successfully using a house price prediction dataset. The output predicted by Arduino closely matched that from the Python model, proving correctness and efficiency.

5. Demonstration & Results

The FFNN was trained on a sample dataset to predict house prices based on features such as area, number of rooms, and location index. After training:

- The model was converted to a .h file.

- Deployed to Arduino UNO.

- Inference was performed in real-time on the board.

- Predicted results were displayed via Serial Monitor.

Graphs comparing Python and Arduino predictions were shown to validate the consistency between platforms.

While this demo used a synthetic application, it effectively validated the technical workflow of training → storage → embedded inference.

6. Advantages & Future Work

Advantages

- Runs neural networks on extremely low-resource hardware.

- Fully offline—no cloud or internet required.

- High portability across projects using FFNN.

Future Enhancements

- Extend support for different activation functions.

- Add support for quantized weights (e.g., int16 or fixed-point).

- Introduce support for multi-class classification problems.

- Build an auto-compiler or uploader for Arduino.

- Integrate with real-time sensor data for live prediction.

- Use SBCs like Raspberry Pi for training/updating on-the-go (semi-online learning).

7. Conclusion

This project successfully demonstrates that deep learning—specifically FFNNs—can be practically implemented on microcontrollers like Arduino UNO. While limited in scope, it lays the groundwork for low-cost intelligent hardware capable of inference in real-world embedded systems.

The framework is lightweight, modular, and avoids unnecessary complexity, aligning well with the constraints of real embedded applications.

Let me know if you want this exported as a PDF or formatted into a Word document.